#### Memory Seat Module - Driver C1

| Connector Part Information |  | - 12020557 - 3-Way F M/P 480 (BLK) |  |
| --- | --- | --- | --- |
| Pin | Wire Color | Circuit No. | Function |
| A | ORN | 2740 | Battery Positive Voltage |
| B | BLK | 1350 | Ground |
| C | ORN | 3540 | Battery Positive Voltage |

#### Memory Seat Module - Driver C2

| Connector Part Information |  | - 15332153 - 8-Way F GT 150 (BLK) |  |
| --- | --- | --- | --- |
| Pin | Wire Color | Circuit No. | Function |
| A | DK GRN | 286 | Driver Seat Front Vertical Motor Up Control |
| B | DK BLU | 287 | Driver Seat Front Vertical Motor Down Control |
| C | LT BLU | 283 | Driver Seat Rear Vertical Motor Down Control |
| D | YEL | 282 | Driver Seat Rear Vertical Motor Up Control |
| E | LT GRN | 284 | Driver Seat Horizontal Motor Rearward Control |
| F | TAN | 285 | Driver Seat Horizontal Motor Forward Control |
| G | LT BLU | 277 | Driver Seat Recline Motor Rearward Control |
| H | LT GRN | 276 | Driver Seat Recline Motor Forward Control |

#### Memory Seat Module - Driver C3

| Connector Part Information |  | - 12110206 - 24-Way F M/P 100 (LT BLU) |  |
| --- | --- | --- | --- |
| Pin | Wire Color | Circuit No. | Function |
| A1 | YEL | 2080 | Low Reference |
| A2 | ORN | 940 | 5 Volt Reference |
| A3 | BLK | 1062 | Driver Seat Lumbar Vertical Position Sensor Signal |
| A4 | YEL | 1063 | Driver Seat Lumbar Horizontal Position Sensor Signal |
| A5-A6 | -- | -- | Not Used |
| A7 | ORN | 840 | 5 Volt Reference |
| A8 | BLK | 850 | Ground |
| A9 | WHT/BLK | 570 | Driver Seat Recline Motor Position Sensor Signal |
| A10 | DK GRN | 569 | Horizontal Seat Motor Position Sensor Signal |
| A11 | TAN | 568 | Rear Vertical Seat Motor Position Sensor Signal |
| A12 | BRN/WHT | 557 | Front Vertical Seat Motor Position Sensor Signal |
| B1 | YEL/BLK | 2079 | Heated Seat Thermistor Signal |
| B2 | BLK | 950 | Low Reference |
| B3 | PPL | 767 | Driver Seat Lumbar Motor Down Control |
| B4 | TAN | 768 | Driver Seat Lumbar Motor Up Control |
| B5-B6 | -- | -- | Not Used |
| B7 | RED/WHT | 2077 | Heated Seat Back Element Supply Voltage |
| B8 | RED/WHT | 2077 | Heated Seat Cushion Element Supply Voltage |
| B9 | DK BLU | 611 | Driver Seat Lumbar Motor Forward Control |
| B10 | PNK | 610 | Driver Seat Lumbar Motor Rearward Control |
| B11-B12 | -- | -- | Not Used |

#### Memory Seat Module - Driver C4

| Connector Part Information |  | - 12110088 - 24-Way F M/P 100 (LT GRY) |  |
| --- | --- | --- | --- |
| Pin | Wire Color | Circuit No. | Function |
| A1-A3 | -- | -- | Not Used |
| A4 | DK GRN/WHT | 1270 | Power Seat Recline Rearward Switch Signal |
| A5 | GRY/BLK | 1269 | Power Seat Recline Forward Switch Signal |
| A6 | LT GRN | 1523 | Power Seat Horizontal Rearward Switch Signal |
| A7 | TAN | 1522 | Power Seat Horizontal Forward Switch Signal |
| A8 | LT BLU | 1521 | Power Seat Rear Vertical Down Switch Signal |
| A9 | YEL | 1519 | Power Seat Rear Vertical Up Switch Signal |
| A10 | DK BLU | 1520 | Power Seat Front Vertical Down Switch Signal |
| A11 | DK GRN | 1518 | Power Seat Front Vertical Up Switch Signal |
| A12 | LT GRN | 2078 | Heated Seat Cushion Element Control |
| B1 | -- | -- | Not Used |
| B2 | BRN/WHT | 1048 | SCM (Seat) Class 2 Serial Data |
| B3 | -- | -- | Not Used |
| B4 | GRY | 961 | Battery Positive Voltage |
| B5 | RED/WHT | 240 | Battery Positive Voltage |
| B6-B7 | -- | -- | Not Used |
| B8 | TAN/WHT | 1067 | Driver Seat Lumbar Down Switch Signal |
| B9 | BRN/WHT | 1066 | Driver Seat Lumbar Up Switch Signal |
| B10 | RED | 1065 | Driver Seat Lumbar Rearward Switch Signal |
| B11 | PNK | 1064 | Driver Seat Lumbar Forward Switch Signal |
| B12 | LT GRN | 2078 | Heated Seat Back Element Control |