# A2a Instrument cluster control module

## Connector overview

| Number | X-pin, color | Description |
| --- | --- | --- |
| X16 | 18-pin, black | Component connector Instrument cluster control module |

## Pin assignments X16

| Pin | Type | Description / Signal type | Connection / Measuring notes |
| --- | --- | --- | --- |
| 1 |  | Not occupied |  |
| 2 | A | Supply, immersed tube indicator | Electric fuel pump |
| 3 | A | Supply, immersed tube indicator | Fuel level sensor, left tank half |
| 4 | A | Power supply, outside temperature sensor | Outside temperature sensor |
| 5 | E | Signal, outside temperature sensor | Outside temperature sensor |
| 6 | A/E | CAN bus high | CAN-bus connector |
| 7 | A/E | CAN bus low | CAN-bus connector |
| 8 |  | Not occupied |  |
| 9 | E | Supply Instrument cluster control module | Fuse F13 |
| 10 |  | Not occupied |  |
| 11 | E | Signal, coolant level | Coolant level switch |
| 12 | E | Signal, washer fluid level | Wash water level switch |
| 13 | E | Signal, parking brake switch | Parking brake switch |
| 14 |  | Not occupied |  |
| 15 | E | Signal, immersed tube indicator | Electric fuel pump |
| 16 | E | Signal, immersed tube indicator | Fuel level sensor, left tank half |
| 17 | E | Not occupied |  |
| 18 | M | Ground | Ground connector |