| Connector | Pin + Colour | Function | Connects to |
| --- | --- | --- | --- |
| C101 | 2, Red | Supply for injectors + coils 2,4,6,8 | Ignition switch (Ign on) |
| C101 | 3, Light Green | Supply for injectors + coils 1,3,5,7 | Ignition switch (Ign on) |
| C101 | 5, Black/Red | Gnd | Gnd |
| C101 | 6, Pink/Blue | Supply for Skipshift+ Reverse lockout | Ignition switch (Ign on) |
| C101 | 7, Green | Supply for AC clutch | Mazda relay (if going Mazda route) |
| C206 | 2, Black | Gnd | Gnd |
| C206 | 5, Blue/White | High speed fan relay control | Gnd for Mazda relay coil |
| C206 | 6, Orange | Supply for PCM | Battery |
| C206 | 7, Brown/Blue | Reverse light switch | Mazda loom |
| C206 | 8, Orange | Supply for PCM | Ignition switch (Ign on) |
| C206 | 13, Pink | Supply for O2 sensors | Ignition switch (Ign on) |
| C206 | 19, Brown/White | Mil control | Mazda engine check lamp |
| C206 | 22, LightGreen | Reverse light switch | Mazda loom |
| C206 | 23, Green/White | Fuel Pump Relay Control | Fuel pump relay coil |
| C206 | 25, Brown | Tach signal wire | Mazda Tach |
| C206 | 26, Yellow | RFA Class 2 Serial Data | OBDII connector |
| C2 PCM (GN) | 21, Blue/White | ISS high signal | "T" to DD IG5 |