| Connector Part Information |  | - PCM Connector C1 Assembly 12191489 - TPA (BLU) 12176408 - Connector Cover 12191108 |  |
| --- | --- | --- | --- |
| Pin | Wire Color | Circuit No. | Function |
| 1 | BLK | 451 | PCM Ground |
| 2 | LT GRN | 1867 | Crankshaft Position Sensor B+ Supply |
| 3 | PNK/BLK | 1746 | Injector 3 Control |
| 4 | LT GRN/BLK | 1745 | Injector 2 Control |
| 5-7 | -- | -- | Not Used |
| 8 | GRA | 596 | TP Sensor 5V Reference |
| 9-10 | -- | -- | Not Used |
| 11 | LT BLU | 1876 | Knock Sensor Signal Rear |
| 12 | DK BLU/WHT | 1869 | Crankshaft Position Sensor Signal |
| 13-16 | -- | -- | Not Used |
| 17 | DK BLU | 1225 | Transmission Range Signal B |
| 18 | RED | 1226 | Transmission Range Signal C |
| 19 | PNK | 439 | Ignition Positive Voltage |
| 20 | ORN | 340 | Battery Positive Voltage |
| 21 | YEL/BLK | 1868 | Crankshaft Position Sensor Reference Low |
| 22 | -- | -- | Not Used |
| 23 | GRA | 720 | Fuel Tank Pressure Sensor/Fuel Tank Sender Ground |
| 24 | -- | -- | Not Used |
| 25 | TAN | 1671 | HO2S Signal Low Bank 2 Sensor 2 |
| 26 | TAN | 1667 | HO2S Signal Low Bank 2 Sensor 1 |
| 27 | -- | -- | Not Used |
| 28 | TAN/WHT | 1669 | HO2S Signal Low Bank 1 Sensor 2 |
| 29 | TAN/WHT | 1653 | HO2S Signal Low Bank 1 Sensor 1 |
| 30-31 | -- | -- | Not Used |
| 32 | GRY | 48 | Clutch Pedal Position Switch Signal |
| 33 | PPL | 420 | TCC Brake Switch |
| 34 | ORN/BLK | 434 | PNP Switch Signal |
| 35 | -- | -- | Not Used |
| 36 | BLK | 1744 | Injector 1 Control |
| 37 | YEL/BLK | 846 | Injector 6 Control |
| 38-39 | -- | -- | Not Used |
| 40 | BLK | 451 | PCM Ground |
| 41 | BLK | 407 | EGR Pintle Position Sensor Ground |
| 42 | DK GRN | 335 | Engine Cooling Fan Relay 1 Control |
| 43 | RED/BLK | 877 | Injector 7 Control |
| 44 | LT BLU/BLK | 844 | Injector 4 Control |
| 45 | GRA | 474 | A/C Refrigerant Pressure Sensor 5V Reference |
| 46 | GRA | 474 | Fuel Tank Pressure Sensor 5V Reference |
| 47 | GRA | 416 | EGR Pintle Position Sensor 5V Reference |
| 48 | GRA | 416 | MAP Sensor 5V Reference |
| 49-50 | -- | -- | Not Used |
| 51 | DK BLU | 496 | Knock Sensor Signal Front |
| 52 | -- | -- | Not Used |
| 53 | BLK | 407 | Transmission Temperature Sensor Ground |
| 54 | ORN/BLK | 407 | MAP Sensor Ground |
| 55 | BRN | 1456 | EGR Pintle Position Sensor Signal |
| 56 | -- | -- | Not Used |
| 57 | ORN | 340 | Battery Positive Voltage |
| 58 | DK GRN | 1049 | Serial Data |
| 59 | -- | -- | Not Used |
| 60 | BLK | 452 | TP Sensor Ground |
| 61 | PNK/BLK | 632 | Camshaft Position Sensor Reference Low |
| 62-64 | -- | -- | Not Used |
| 65 | PPL | 1670 | HO2S Signal High Bank 2 Sensor 2 |
| 66 | PPL | 1666 | HO2S Signal High Bank 2 Sensor 1 |
| 67 | -- | -- | Not Used |
| 68 | PPL/WHT | 1668 | HO2S Signal High Bank 1 Sensor 2 |
| 69 | PPL/WHT | 1665 | HO2S Signal High Bank 1 Sensor 1 |
| 70 | BRN | 1174 | Low Oil Level Switch |
| 71-72 | -- | -- | Not Used |
| 73 | BRN/WHT | 633 | Camshaft Position (CMP) Sensor Signal |
| 74 | YEL | 410 | Engine Coolant Temperature (ECT) Sensor Signal |
| 75 | -- | -- | Not Used |
| 76 | BLK/WHT | 845 | Injector 5 Control |
| 77 | DK BLU/WHT | 878 | Injector 8 Control |
| 78 | -- | -- | Not Used |
| 79 | GRA or WHT | 587 or 687 | Skip Shift Solenoid Control (M/T) or 3-2 Shift Solenoid Control (A/T) |
| 80 | BLK | 407 | Engine Coolant Temperature (ECT) Sensor Ground |