| C100 |  |  |  | question = |
| --- | --- | --- | --- | --- |
| A | PNK |  | Hot in Run and Start (IGN/INJ Bank 1 15A) | install to Ignition power ? |
| B | DK GRN/WHT | PCM 43 | A/C Clutch Relay Control |  |
| C | DK GRN | PCM 18 | A/C Clutch Status |  |
| D | GRY |  | TPS +5v to TCS | remove ? |
| E | DK BLU |  | TPS signal to TCS | remove ? |
| G | PNK |  | Hot with IGN 1 Relay Engaged (Tranny/EGR 15A) | install to Ignition power ? |
| H | DK BLU | PCM 33 | Cooling Fan 2 and 3 Relay Control |  |
| J | DK GRN | PCM 42 | Cooling Fan 1 Relay Control |  |
| K | BLK |  | TPS sensor low to TCS | remove ? |
| C101 Plug |  |  |  |  |
| A | YEL/BLK |  | Low Coolant Sensor |  |
| B | PNK |  | Hot in Run and Start (IGN/INJ Bank 2 15A) | install to Ignition power ? |
| C | BRN | PCM 36 | Air Pump Relay Control |  |
| D | DK GRN/WHT | PCM 9 | Fuel Pump Relay Control |  |
| E | PNK/BLK | PCM 19 | Ignition Positive Voltage for PCM | install to Ignition power ? |
| G | ORN | PCM 57 | Battery Positive Voltage for PCM |  |
| H | BLK/WHT |  | Ground |  |
| K | GRY/BLK | PCM 53 | TCS Spark Retard Signal to Brake Control Module |  |
| C105 Plug |  |  |  |  |
| B | PNK |  | Hot with IGN 1 Relay Engaged (TCC Stop Lamp Switch/MAF/O2 20A) | install to Ignition power ? |
| D | DK GRN | PCM 37 | Cruise Control Inhibit |  |
| F | WHT | PCM 13 | Cruise Engaged Signal |  |
| G | WHT | PCM 10 | Tach out to Brake Control Module |  |
| H | PPL | PCM 4 | AIR Solenoid Relay Control |  |