| 2005 GTO LS2 Pinouts Connector J1 |  |  |  |
| --- | --- | --- | --- |
| Pin # |  |  |  |
| 1 | bus + | Tan/Blk | Data Lines (not used) |
| 2 | Bus - | Tan | Data Lines (not used) |
| 3 | Unused |  |  |
| 4 | Mil Cntrl | Brn/Wht | MIL Light |
| 5 | Unused |  |  |
| 6 | Unused |  |  |
| 7 | Low Ref | Brown | Gas Pedal Pin 1 |
| 8 | Low Ref | Purple | Gas Pedal Pin 7 |
| 9 | Unused |  |  |
| 10 | Eng Cntrl Rel Coil Ctrl | Brown | Engine Controls Relay |
| 11 | Unused |  |  |
| 12 | Unused |  |  |
| 13 | Starter Relay Coil | Yellow | Starter Relay Coil |
| 14 | Ign 1 Voltage | Pink | From 15 Amp Engine sensors fuse and other side of Starter relay coil |
| 15 | Unused |  |  |
| 16 | Unused |  |  |
| 17 | Unused |  |  |
| 18 | Unused |  |  |
| 19 | Ign 3 Voltage | Orange | From ignition switch Pin 15, 12 volts in Run & Start |
| 20 | Batt + Voltage | Orng/Blk | From 15 Amp Eng control fuse (hot at all times) |
| 21 | VSS signal | Purpl/Wht | Speed signal to electronic speedometer (unsure of this one?!?!?!?) |
| 22 | APP Sen 1 Signal | Blue | Gas Pedal Pin 5 |
| 23 | Unused |  |  |
| 24 | Sensor Signal | Green | To fuel tank pressure sensor pin B (delete in tuning) |
| 25 | Unused |  |  |
| 26 | A/C Pressure Signal | Green/blk | A/C pressure sensor pin C |
| 27 | Unused |  |  |
| 28 | Unused |  |  |
| 29 | 5 Volt Ref | Tan | Gas pedal Pin 6 |
| 30 | A/C Pressure Signal | Green | A/C pressure sensor pin B |
| 31 | 5 Volt Ref | Gray | To fuel tank pressure sensor pin C (delete in tuning) |
| 32 | SW Signal | Blu/Red | To exterior lights system |
| 33 | Unused |  |  |
| 34 | Unused |  |  |
| 35 | 5 Volt Ref | White/Blk | Gas pedal Pin 2 |
| 36 | Fan Control Relays | Orng/Blk | Cooling Fans system |
| 37 | Fuel Level Sensor | Purple | To fuel level sensor in fuel tank |
| 38 | SW Signal | Gray | To exterior lights system |
| 39 | Unused |  |  |
| 40 | Unused |  |  |
| 41 | APP Sen 2 Signal | Lt Blue | Gas pedal Pin 3 |
| 42 | Reverse Lock Out | Yellow | Reverse Lock Out Solenoid (unsure of this one!?!?!?) |
| 43 | CPP Sw Signal | Prpl/Wht | To cruise control system |
| 44 | Unused |  |  |
| 45 | Fuel Pump Relay | Green/Wht | Fuel Pump Relay Pin 1, use to turn on your fuel pump relay |
| 46 | Release Sig | Brown | To exterior lights system |
| 47 | Unused |  |  |
| 48 | Engine RPM Signal | Brown | RPM output to tach |
| 49 | Unused |  |  |
| 50 | Unused |  |  |
| 51 | Park Neutral SW | Green/Blu | To neutral safety switch |
| 52 | Crank Voltage | Purple | To ignition switch, start position (hot in start only) |
| 53 | Unused |  |  |
| 54 | Ign Coil 1 Control | White | To ignition coil Cylinder # 1 |
| 55 | A/C Pressure Signal | Tan | A/C pressure sensor pin A |
| 56 | Fan Control Relays | Blu/Wht | Cooling Fans system |