VACUUM TESTING/READINGS

NORMAL RANGE MOST 15/22 INCHES hg / ELEVATION INCREASE PLUS 1 INCH-  PER ONE THOUSAND FEET GAIN IN ALTITUDE.

A  CAM WITH NARROW LSA REDUCES VACUUM PRESSURE.

1.WIDE FLUCTUATIONS AT HIGHER RPM/WEAK VALVE SPRINGS.

2.STEADY IRREGULAR DROP/BURNT VALVES OR VALVE HOLDING OPEN/MISFIRE.

3.NORMAL RANGE/BOUNCING NEEDLE/INTERNAL ISSUES TIMING/AIR FUEL/RINGS/VALVES/HEAD GASKET.

4.INTERMITTENTLY DROPPING AT IDLE/STICKING  VALVE. DO COMPRESSION OR LEAK DOWN TEST TO VERIFY PROBLEM.

5.LOW STEADY VACUUM READING/INTAKE GASKET/CARB/THROTTLE BODY/VACUUM HOSE/LATE TIMING/INCORRECT CAM TIMING. USUALLY 3 TO 8 INCHESBELOW NORMAL.

6.RAPID VIBRATION WITH EXHAUST SMOKE/USUALLY 4 INCHES/INDICATES WORN VALVE GUIDES.

7.SLIGHT FLUCTUATIONS OF 1 INCH PLUS/MINUS/IGNITION ISSUES/CHECK TUNE/USE ANALYZER.

8.STEADY DROP TO ZERO/RESTRICTED EXHAUST/CHOCKED/BLOCKED/LACK OF POWER.

9.IDLE 1800/2500 RPM.S/RELEASE THROTTLE/SHOULD FALL TO IDLE/NEEDLE SHOULD

JUMP AND THEN RETURN TO NORMAL READING. IF NOT CHECK VALVES/RINGS/ETC.

ALWAYS USE THIS AS A TOOL ONLY!!! COMPRESSION/LEAKDOWN/ANALYZER TEST WILL VERIFY PROBLEMS ALONG WITH VISUAL OBSERVATIONS AND EXPERTISE.

JIM RAVENCRAFT