LS6 Camshaft: Intake Comparison
(All lift figures are tappet lift)

| year PN | int. lift | int. dur at .004 | int. dur. at .050 | int. open at .004 | in. close at .004 | in. open at .050 | in. close at .050 | int. CL | int. area | int. area increase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MY01 12560950 | 13.34 m .525 in | 270° | 204° | 9° BTDC | 81° ABDC | 18 ATDC | 42 ABDC | 118° ATDC | 1862.9 mm/deg. |  |
| MY02 12565308 | 14.01 m .551 in | 267° | 204° | 7° BTDC | 80° ABDC | 19° ATDC | 43 ABDC | 120° ATDC | 1936.9 mm/deg. | 4% |

LS6 Camshaft: Exhaust Comparison
(All lift figures are tappet lift)

| year PN | exh. lift | exh. dur. at. .004 | exh. dur. .050 | ex. open .004 | ex. close .004 | ex. open .050 | ex. close .050 | exh. CL | exh. area | ex. area increase |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| MY01 12560950 | 13.33 m .525 in | 275° | 211° | 65° BBDC | 30° ATDC | 37 BBDC | 6 BTDC | 114° BTDC | 1914.6 mm/deg. |  |
| MY02 12565308 | 13.91 m .547 in | 282° | 218° | 69° BBDC | 33° ATDC | 42 BBDC | 4 BTDC | 115° BTDC | 2046.6 mm/deg. | 8% |