- Fastener Tightening Specifications F Car

| Application | Specification |  |
| --- | --- | --- |
|  | Metric | English |
| Accelerator Control Cable Bracket Bolts | 10 N·m | 89 lb in |
| Air Injection Reaction (AIR) Pipe-to-Exhaust Manifold Bolts | 20 N·m | 15 lb ft |
| Camshaft Retainer Bolts | 25 N·m | 18 lb ft |
| Camshaft Sensor Bolt | 25 N·m | 18 lb ft |
| Camshaft Sprocket Bolts | 35 N·m | 26 lb ft |
| Connecting Rod Bolts - Early Design - First Pass | 20 N·m | 15 lb ft |
| Connecting Rod Bolts - Early Design - Final Pass | 60 degrees |  |
| Connecting Rod Bolts - Later Design - First Pass | 20 N·m | 15 lb ft |
| Connecting Rod Bolts - Later Design - Final Pass | 75 degrees |  |
| Coolant Temperature Gage Sensor | 20 N·m | 15 lb ft |
| Crankshaft Balancer Bolt - Installation Pass-to Ensure the Balancer is Completely Installed | 330 N·m | 240 lb ft |
| Crankshaft Balancer Bolt - First Pass-Install a NEW Bolt After the Installation Pass and Tighten as Described in the First and Final Passes | 50 N·m | 37 lb ft |
| Crankshaft Balancer Bolt - Final Pass | 140 degrees |  |
| Crankshaft Bearing Cap Bolts - Inner Bolts-First Pass in Sequence | 20 N·m | 15 lb ft |
| Crankshaft Bearing Cap Bolts - Inner Bolts-Final Pass in Sequence | 80 degrees |  |
| Crankshaft Bearing Cap Side Bolts | 25 N·m | 18 lb ft |
| Crankshaft Bearing Cap Studs - Outer Studs-First Pass in Sequence | 20 N·m | 15 lb ft |
| Crankshaft Bearing Cap Studs - Outer Studs-Final Pass in Sequence | 53 degrees |  |
| Crankshaft Oil Deflector Nuts | 25 N·m | 18 lb ft |
| Crankshaft Position Sensor Bolt | 25 N·m | 18 lb ft |
| Cylinder Head Bolts - First Pass all M11 Bolts in Sequence | 30 N·m | 22 lb ft |
| Cylinder Head Bolts - Second Pass all M11 Bolts in Sequence | 90 degrees |  |
| Cylinder Head Bolts - Final Pass all M11 Bolts in Sequence-Excluding the Medium Length Bolts at the Front and Rear of Each Cylinder Head | 90 degrees |  |
| Cylinder Head Bolts - Final Pass M11 Medium Length Bolts at the Front and Rear of Each Cylinder Head in Sequence | 50 degrees |  |
| Cylinder Head Bolts - M8 Inner Bolts in Sequence | 30 N·m | 22 lb ft |
| Cylinder Head Coolant Plug | 20 N·m | 15 lb ft |
| Cylinder Head Core Hole Plug | 20 N·m | 15 lb ft |
| Engine Block Coolant Drain Plugs | 60 N·m | 44 lb ft |
| Engine Block Heater | 40 N·m | 30 lb ft |
| Engine Block Oil Gallery Plugs | 60 N·m | 44 lb ft |
| Engine Coolant Air Bleed Pipe Bolts and Studs | 12 N·m | 106 lb in |
| Engine Flywheel Bolts - First Pass | 20 N·m | 15 lb ft |
| Engine Flywheel Bolts - Second Pass | 50 N·m | 37 lb ft |
| Engine Flywheel Bolts - Final Pass | 100 N·m | 74 lb ft |
| Engine Front Cover Bolts | 25 N·m | 18 lb ft |
| Engine Rear Cover Bolts | 25 N·m | 18 lb ft |
| Engine Service Lift Bracket M10 Bolts | 50 N·m | 37 lb ft |
| Engine Service Lift Bracket M8 Bolt | 25 N·m | 18 lb ft |
| Engine Valley Cover Bolts | 25 N·m | 18 lb ft |
| Exhaust Manifold Bolts - First Pass | 15 N·m | 11 lb ft |
| Exhaust Manifold Bolts - Final Pass | 25 N·m | 18 lb ft |
| Exhaust Manifold Heat Shield Bolts | 9 N·m | 80 lb in |
| Fuel Injection Fuel Rail Bolts | 10 N·m | 89 lb in |
| Ignition Coil-to-Bracket Bolts | 12 N·m | 106 lb in |
| Ignition Coil Bracket-to-Valve Rocker Arm Cover Bolts | 12 N·m | 106 lb in |
| Intake Manifold Bolts - First Pass in Sequence | 5 N·m | 44 lb in |
| Intake Manifold Bolts - Final Pass in Sequence | 10 N·m | 89 lb in |
| Knock Sensors | 20 N·m | 15 lb ft |
| Oil Filter | 30 N·m | 22 lb ft |
| Oil Filter Fitting | 55 N·m | 40 lb ft |
| Oil Level Indicator Tube Bolt | 25 N·m | 18 lb ft |
| Oil Level Sensor | 13 N·m | 115 lb in |
| Oil Pan Baffle Bolts | 12 N·m | 106 lb in |
| Oil Pan Closeout Cover Bolt - Left Side | 12 N·m | 106 lb in |
| Oil Pan Closeout Cover Bolt - Right Side | 12 N·m | 106 lb in |
| Oil Pan Cover Bolts | 12 N·m | 106 lb in |
| Oil Pan Drain Plug | 25 N·m | 18 lb ft |
| Oil Pan M8 Bolts - Oil Pan-to-Engine Block and Oil Pan-to-Front Cover | 25 N·m | 18 lb ft |
| Oil Pan M6 Bolts - Oil Pan-to-Rear Cover | 12 N·m | 106 lb in |
| Oil Pressure Sensor | 20 N·m | 15 lb ft |
| Oil Pump-to-Engine Block Bolts | 25 N·m | 18 lb ft |
| Oil Pump Cover Bolts | 12 N·m | 106 lb in |
| Oil Pump Relief Valve Plug | 12 N·m | 106 lb in |
| Oil Pump Screen Nuts | 25 N·m | 18 lb ft |
| Oil Pump Screen-to-Oil Pump Bolt | 12 N·m | 106 lb in |
| Oxygen Sensor | 42 N·m | 31 lb ft |
| Positive Crankcase Ventilation (PCV) System Strap Nut - at Right Front Engine Coolant Air Bleed Pipe Stud | 12 N·m | 106 lb in |
| Spark Plugs - Cylinder Heads-New | 20 N·m | 15 lb ft |
| Spark Plugs - all Subsequent Installations | 15 N·m | 11 lb ft |
| Throttle Body Bolts | 12 N·m | 106 lb in |
| Valve Lifter Guide Bolts | 12 N·m | 106 lb in |
| Valve Rocker Arm Bolts | 30 N·m | 22 lb ft |
| Valve Rocker Arm Cover Bolts | 12 N·m | 106 lb in |
| Water Inlet Housing Bolts | 15 N·m | 11 lb ft |
| Water Pump Bolts - First Pass | 15 N·m | 11 lb ft |
| Water Pump Bolts - Final Pass | 30 N·m | 22 lb ft |
| Water Pump Cover Bolts | 15 N·m | 11 lb ft |