| ..GM LS-1 (1997- and up) .....STREET/STRIP GRINDS Description | RPM Range | Duration...............Lift |  |  |  | Lope Sep Int C/L | Opens/Closes @ .050 | Camshaft Part Number | Cam & Lifters Part Number | Valve Lash | Grind Number | Lifters | Springs | Retainers | Locks | Timing Chain | Rocker Arms | Valvetrain Kit* |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  | Adv In/Exh | @ .050" In/Exh | @ Valve In/Exh | @ Lobe In/Exh |  |  |  |  |  |  |  |  |  |  |  |  |  |
| HYDRAULIC ROLLER, Good idle, excellent low end torque and mid-range horsepower. | 1800-4800 | 270° 279° | 212° 221° | .510" .510" | .300" .300" | 116° Sep 112° Int C/L | -06°BTC 38°ABC / 50.5°BBC -9.5°ATC | 55002LUN $343.00 Bottom of Form Top of Form Bottom of Form | N/A | HYD | RRS1 212-221 | N/A | 74150 121.00 Bottom of Form Top of Form Bottom of Form | 76120 $147.00 Bottom of Form Top of Form Bottom of Form | Use Factory | N/A | N/A | N/A |
| HYDRAULIC ROLLER, Fair idle. High performance cam. Lots of mid-range torque and upper horsepower. | 2000-5000 | 282° 287° | 218° 221° | .526" .534" | .309" .314" | 116° Sep 112° Int C/L | -03°BTC 41°ABC / 50.5°BBC -9.5°ATC | 55003LUN $343.00 Bottom of Form Top of Form Bottom of Form | N/A | HYD | RRS1 218-221 | N/A | 74150 121.00 Bottom of Form Top of Form Bottom of Form | 76120 $147.00 Bottom of Form Top of Form Bottom of Form | Use Factory | N/A | N/A | N/A |

Bottom of Form