Guerra Motorsports Custom Camshaft Recommendation

Customer name: Chris Burken

Internet screen name: ls1tech- “burken01”

Street Address: 620 s. Alvy st

City, State, Zip, Country: Anaheim, Ca 92802

Contact phone number: (714) 600-2086

E-mail address: burken46@yahoo.com

Cam recommendation: 1998 Camaro SS with TSP forged LS15.7L short block, 62cc stock LS6 heads, LS6 intake with 78mm TB, auto trans with 4000 stall, 3.73 gear, 1 3/4" long tube headers with no cats, 6600 shift point, 6800 rpm rev limiter.

227/230 .614”/.609” 111LSA +2 advance (with 1.7 ratio rockers)

Comp LSL/LXL lobes 54-000-11 3 bolt cam core

13017R/13159R HR111LSA +2 advance (109 installed intake centerline)

Cam Valve Events; 0.006; 0.050; 0.200. Intake Duration - ID; 277; 227; 151; 13017R LSL lobe .614". Exhaust Duration - ED; 282; 230; 151; 13159R LXL lobe .609". Lobe Center Angle - LSA; 111; 111; 111. Intake Centerline - ICL; 109; 109; 109. Intake Valve opens - IVO; 29.5; 4.5; -33.5; BTDC (- indicates ATDC). Intake Valve closes - IVC; 67.5; 42.5; 4.5; ABDC. Exhaust Valve Opens - EVO; 74; 48; 8.5; BBDC. Exhaust Valve Closes - EVC; 28; 2; -37.5; ATDC (- indicates BTDC). Exhaust Centerline - ECL; 113; 113; 113. Overlap; 57.5; 6.5; -71; degrees.

Static/Dynamic compression with new cam: 11.0:1, 8.59:1.

Recommended valve springs: TEA .660" duals with Ti retainers.

Recommended pushrod length: 7.425” w/LS7 or stock lifters, stock rockers.

Ordering instructions:

Contact Geoff at Engine Power Systems: geoff@engpwrsys.com or 225-751-8500.

Submit this order form as an e-mail attachment or by fax: 225-751-8581 (fax).

Geoff will contact you for payment information.

E.P.S. can verify that your custom camshaft is ground properly on their Cam Pro Plus and you will receive a printout of the exact as-built specifications.

114 Spokane Ave. ~  Victoria, Tx. ~  77904

361.576.5917 office ~  361.576.5928 fax

www.guerragroup.com

Copyright © The Guerra Group 2006. All transfer or reproduction prohibited by law without express written permission.