BASIC SETUP FOR VE/MAF TUNING

(Note: This is on my 2002 Camaro Z28 (GEN III), some info me be missing for other apps)

CLOSED LOOP SPEED DENSITY (VE TUNING w/ NARROW BAND)

- Copy original tune file to CLSD.hpt

- Vehicles with Secondary VE tables need to copy values from Primary VE to the Secondary VE

- Copy High Octane Spark Table to Low Octane Spark Table

- Disable SES light for P0101, P0102, P0103 (Uncheck SES code, MIL on First Error)

- MAF Fail High Frequency to 0 (this disables MAF and sets pcm to speed density mode)

- Engine-->Fuel Control-->

  - Fuel Cutoff--> DFCO Enable Normal Set enable temp to 284F (from stock -22)

  - Long Term Fuel Trims to Disabled (STFT still active)

  - Turn off COT (Catalytic Overtemp Protection)

- Reset Fuel Trims using VCM Controls in Scanner

- Take a drive and try to hit as many cells on the VE table as possible but stay out of PE, I like to get a cell count of at least 25

- Use the average fuel trims from the histogram and Copy/Paste Special % Half onto the VE table in tune

- Might take several iterations to get your trims near 0%, try to get at least within +-2% (be sure to reset fuel trims between each run)

- You could also get fancy with the histo and filter out any PE data by logging commanded AFR and using a filter of [SENS.121]>14.6

- Once VE table is adjusted copy it back to your original tune. (easier then removing all these edits)

OPEN LOOP SPEED DENSITY (VE TUNING w/ WIDEBAND)

- Copy original tune (which now has VE close by narrowband)tune to OLSD.hpt

- Vehicles with Secondary VE tables need to copy values from Primary VE to the Secondary VE

- Copy High Octane Spark Table to Low Octane Spark Table

- Disable SES light for P0101, P0102, P0103 (Uncheck SES code, MIL on First Error)

- MAF Fail High Frequency to 0 (this disables MAF and sets pcm to speed density mode)

- Multiply VE table from 4000rpm and up all map values by 1.15 (to insure enough fuel at higher load/rpm, since lower ve table is already close from narrowband tuning)

- Engine-->Fuel Control-->

  - Fuel Cutoff--> DFCO Enable Normal Set enable temp to 284F (from stock -22)

  - Power Enrich-->EQ ratio vs. RPM  set to your desired a/f across the board (i.e. 1.13 = 13.0 afr)

  - OL&CL--> EQ Ratio set to 1.00 for 158F and up all map.  This is 14.7:1 air fuel ratio.

  - Long Term Fuel Trims to Disabled

  - Closed Loop Enable ECT-vs-IAT to 284°F (ensure you go into open loop)

  - Turn off COT (Catalytic Overtemp Protection)

- Reset Fuel Trims using VCM Controls in Scanner

- Take a drive and try to hit as many cells on the VE table as possible, I like to get a cell count of at least 25

- Use the Wideband % Error from the histogram and Copy/Paste Special % Half onto the VE table in tune

- Might take several iterations to get your % Error near 0%

- Once VE table is adjusted copy it back to your original tune. (easier then removing all these edits)

CLOSED LOOP MAF (MAF TUNING w/ NARROW BAND)

- Copy original tune (which now has VE completely tuned) to CLMAF.hpt

- Copy High Octane Spark Table to Low Octane Spark Table

- Engine-->Fuel Control-->

  - Fuel Cutoff--> DFCO Enable Normal Set enable temp to 284F (from stock -22)

  - Long Term Fuel Trims to Disabled (STFT still active)

  - Power Enrich-->EQ ratio vs. RPM  set to your desired a/f across the board (i.e., 1.13 = 13.0 afr)

  - Turn off COT (Catalytic Overtemp Protection)

- Engine-->Airflow-->Dynamic Airflow

High RPM Disable set to 500 (stock = 4000) this will set all airflow measurments over 500 rpm via MAF only, no VE table lookup

- Reset Fuel Trims using VCM Controls in Scanner

- Take a drive and try to hit as many cells on the MAF table as possible but staying out of PE, I like to get a cell count of at least 25

- Use the average fuel trims from the histogram and Copy/Paste Special % Half onto MAF table in tune

- Might take several iterations to get your trims near 0%, try to get atleast within 2% (be sure to reset fuel trims between each run)

- You could also get fancy with the histo and filter out any PE data by logging commanded AFR and using a filter of [SENS.121]>14.6

- Once MAF table is adjusted copy it back to your original tune. (easier then removing all these edits)

OPEN LOOP MAF (MAF TUNING w/ WIDEBAND)

- Copy original tune (which now has VE tuned and MAF close by narrowband) to OLMAF.hpt

- Copy High Octane Spark Table to Low Octane Spark Table

- Engine-->Fuel Control-->

  - Fuel Cutoff--> DFCO Enable Normal Set enable temp to 284F (from stock -22)

  - Power Enrich-->EQ ratio vs. RPM  set to your desired a/f across the board (i.e. 1.13 = 13.0 afr)

  - OL&CL--> EQ Ratio set to 1.00 for 158F and up all map.  This is 14.7:1 air fuel ratio.

  - Long Term Fuel Trims to Disabled

  - Closed Loop Enable ECT-vs-IAT to 284°F (ensure you go into open loop)

  - Turn off COT (Catalytic Overtemp Protection)

- Engine-->Airflow-->Dynamic Airflow

High RPM Disable set to 500 (stock = 4000) this will set all airflow measurments over 500 rpm via MAF only, no VE table lookup

- Reset Fuel Trims using VCM Controls in Scanner

- Take a drive and try to hit as many cells on the MAF table as possible but staying out of PE, I like to get a cell count of at least 25

- Use the Wideband % Error from the histogram and Copy/Paste Special % Half

- Might take several iterations to get your trims near 0% (be sure to reset fuel trims between each run)

- Once MAF table is adjusted copy it back to your original tune. (easier then removing all these edits)

- Upload finalized tune back to car

NOTES:

Closed Loop is for tuning with your Narrowband Oxygen Sensors (monitor fuel trims to make adjustments)

Open Loop is for tuning with your Wideband Oxygen Sensor (monitor %Error A/F to make adjustments)

If you know ahead of time your trims are quite aggressive you could start with the closed loop method to get them more in line then move forward to the open loop mode to finalize them, or if you are comfortable with your trims you could just go straight to open loop and stay with that method.

I prefer to take my current tune, make a copy and name it to the mode you are tuning, make the edits to the new copied tune file and then upload it to car.  Do your driving, make you changes, re-upload wash, rinse, and repeat.

Once you get the table you are editing finalized copy it back to your original tune file, that way you don’t have to worry about putting all the adjustment back to their original setting, just copy over your corrected VE for example and your done.  Then make a copy of that for the next mode of tuning and …………. You get the idea